MID EXAM

An instruction is a low-level command that the processor executes. It tells the ARM processor what specific action to perform, such as arithmetic operations, data movement, branching, and more.

MOV : It can optionally update the condition flags based on the value.

ADD, ADDS, SUB, SUBS: It can optionally update the condition flags based on the result.

MUL: These 32 bits do not depend on whether the source register values are considered to be signed values or unsigned values.

CMP: It updates the condition flags based on the result, and discards the result.

Branch: In ARM Assembly, a branch is an instruction used to change the flow of execution in a program — similar to a jump or goto in high-level languages.

| Branch | Condition (Flags) |  | Meaning |
| --- | --- | --- | --- |
| BGT | Z == 0 and N == 0 or V | Branch if greater than (signed) |  |
| BLT | N ≠ V | Branch if less than (signed) |  |
| BGE | N == V | Branch if greater than or equal (signed) |  |
| BLE | Z == 1 or N ≠ V | Branch if less than or equal (signed) |  |
| BEQ | Z == 1 | Branch if equal |  |
| BNE | Z == 0 | Branch if not equal |  |
| BMI | N == 1 | Branch if minus (negative result) |  |

For making a function call in assembly language. Function call uses r0 to r4 GP registers

LDR and STR : These instructions allow ARM assembly programs to interact with data in memory, which is crucial for tasks such as processing arrays, accessing global variables, and manipulating data during program execution.

Data Segment:

The data segment in assembly programming is a section of memory where global variables, constants, and static data are stored.

Key Features of the Data Segment:

• Global Variables: Variables declared in the data segment are typically available throughout the entire program.

• Static Data: Data defined in this segment remains in memory throughout the program’s execution.

• Constants: You can define read-only constants in the data segment.

• Uninitialized Data: Variables that are declared but not initialized are stored in a separate section, typically the BSS (Block Started by Symbol) segment.

Segments in ARM Assembly:

1. Text Segment (.text): Contains the executable instructions (code) of the program.

2. Data Segment (.data): Contains initialized data (like global variables and constants).

3. BSS Segment (.bss): Contains uninitialized variables or variables initialized to zero.

Common Directives Used in Data Segment:

• .word: Allocates a word (typically 4 bytes on 32-bit systems) for storing integers or addresses.

• .asciz: Allocates a null-terminated string.

• .byte: Allocates a single byte of data.

• .hword: Allocates a halfword (2 bytes).

• .space: Allocates a block of memory with uninitialized data

Function: To use the concept of function in arm assembly, we basically use the branch. Specifically, we use BL (Branch with link) opcode when using function.

- org 1000              @program start at memory address 1000/

it is the same as a SUBS instruction, except that the processor discards the result.

Current Program Status Register (CPSR)