Core Sensors and Software:

• DHT11 (Temp/RH)

• Soil-moisture sensor (analog) → ADC

• HC-SR04 ultrasonic for tank level (TRIG/echo)

• MCU: STM32F103C8 (Blue Pill), ST-LINK/V2, breadboard & jumpers.

• LCD 16×2 I²C (PCF8574 @ 0x27, 5 V) with PB6=SCL, PB7=SDA.

• 3× LEDs (G/Y/R) + resistors.

• SG90 Servo

• Software: STM32CubeMX, Keil uVision 5, ST-Link Utility.

LCD UI Screen Show Part :

- ( CSE331

Group: 3 )

- ( Smart Green-

house Manager )

## start Working show data

- ( Temp/RH                                          Data From DHT11

Tank Level : __%)                              Data From HC-SR04

## 10 sec after 3 it show 4, again 10 sec after 4 it show 3

- ( Soil: Dry/ Wet                                   Data From Soil-moisture sensor (analog)

Tank Level : __%)                               Data From HC-SR04

Logic:

If Soil Dry Show Yellow LED and SG90 Servo Start until it Wet

If tank level 10-20cm is show 100-50% and LED green, If tank level 5-10cm is show 50-25% and LED yellow , If tank level 1-5cm is show 25-0% and Red green. When then SG90 Servo Start until it 100%